# Interface Control Document (ICD) – REII Subsystem

Subsystem: Remote Control Dashboard & RF Ground Station

Responsible Engineer: REII 327

## 1. Hardware & Physical Interfaces

This section defines the physical hardware connections between the microcontroller processing units and the RF communication modules.

- Ground Station (Controller):

  - Hardware: Custom STM32F411 dongle.

  - Power Interface: The module will be strictly USB-bus-powered directly from the host PC. It will draw no power from the shared crane system.

  - Data Interface: Communicates with the host PC via native USB/USB bridging. Communicates with the ground-side nRF24L01+ module via hardware SPI.

- Crane Controller (Receiver):

  - Hardware: NWU Engineering ESP32-S3 Dev Board.

  - Data Interface: The crane-side nRF24L01+ module will connect via the "ESP IO header". This header provides direct microcontroller GPIO access required for high-speed SPI communication. (Exact pins to be finalized based on EEII/MCTR usage).

- Mandatory RF Hardware Modification:

  - To prevent system brownouts during 115 mA transmission bursts at PA_MAX, both the STM32F411 and ESP32-S3 nRF24L01+ modules must feature a 10 µF electrolytic (or tantalum) capacitor and a 100 nF ceramic capacitor wired directly in parallel across their VCC and GND pins.

  - Both nRF24L01+ modules must be powered strictly by 3.3V.

## 2. RF Data Link Configuration (Data Link Layer)

To ensure reliable communication in the high-interference campus environment (bypassing overlapping Wi-Fi channels 1, 6, and 11), the nRF24L01+ hardware shall be locked to the following configuration:

- Operating Frequency: Channel 77 (2477 MHz).

- Data Rate: 2000 kbps (ensures minimum time-on-air and 0.00% packet loss at the 5-10m operational range).

- Transmit Power: PA_MAX (Note: must be dropped to PA_LOW for bench testing under 1 meter to avoid LNA saturation).

- Auto-Retry Limit (ARC): 5 retries maximum.

- Auto-Retry Delay (ARD): 500 µs.

- Addressing & CRC: 5-byte address width, 16-bit CRC.

## 3. Application-Layer Protocol

To minimize transmission latency (worst-case radio delay capped at ~4.1 ms) and eliminate complex software-level ARQ handling, the system will utilize the nRF24’s native Enhanced ShockBurst hardware Auto-ACK feature.

- Downstream (PC Dashboard -> Crane): Command instructions will be transmitted as standard data packets.

- Upstream (Crane -> PC Dashboard): Operational telemetry will be transmitted using ACK-payloads. The ESP32-S3 will piggyback sensor and state data onto the automatic hardware acknowledgments triggered by downstream commands.

## 4. Data Payload Structures (TBD - Cross-Discipline Agreement Required)

This section defines the maximum 32-byte data payloads. [ACTION REQUIRED]: The exact bit/byte assignments, data types, and scaling factors must be agreed upon with MCTR (Mechatronics) and EEII (Electrical/Electronics) team members.

### 4.1 Downstream Command Payload (Max 32 Bytes)

[PENDING AGREEMENT WITH MCTR & EEII]

We need to agree on how the PC will send motion commands and configuration data to the crane.

- Crane Motion Commands (MCTR): How are we formatting the open-loop feedforward profiles for anti-sway? Do you need me to send ramp-rates, target velocities, or raw positional waypoints?

- Obstacle Avoidance (MCTR): When the dashboard plans a path around a known obstacle, what specific motion command format does your drive control expect?

- Actuator States (EEII/MCTR): What is the data structure for toggling the electromagnetic attachment mechanism on/off?

- System States: Flags for E-stop, System Reset, and Mode selection (Manual vs. Auto).

### 4.2 Upstream Telemetry ACK-Payload (Max 32 Bytes)

[PENDING AGREEMENT WITH MCTR & EEII]

We need to agree on what live operational telemetry the dashboard is required to monitor.

- Kinematic State (MCTR): What data are you providing back for live monitoring? (e.g., X/Y position coordinates, current velocity, hoist height/spooling status).

- Electrical/Sensor State (EEII): Are you reading temperatures, current draw from the 3-phase induction motor, or limit switch statuses? How are these packed into bytes?

- System Health: Which error codes or fault conditions from the motor/inverter control need to be pushed up to the dashboard?