# REII 327: DESIGN 2026

Sub-Project Report: REII nRF24L01 Link Bring-Up & Characterisation

By:

Riekert Nel

40496325

Submitted in pursuit of the degree

BACHELOR OF ENGINEERING

In

COMPUTER ELECTRONIC ENGINEERING

North-West University Potchefstroom Campus
Abstract

The primary objective of this sub-project was to systematically verify the reliability of an nRF24L01+ RF link prior to committing resources to a custom STM32F411 dongle and application protocol. Testing evaluated hardware stability, channel interference, and optimal data rates for telemetry. This report exclusively details line-of-sight (LOS) packet-loss testing and characterisation. The evaluation confirms the link is highly stable up to 190 meters LOS when operating at 250 kbps, provided specific hardware decoupling modifications are applied.

Table of Contents

1. Problem Statement	5

2. System Setup: Hardware Bring-Up and Software Methods	5

2.1 Method (Software Setup)	5

2.2 Implementation (Hardware Bring-Up)	5

3. Deliverable 1: Packet-Loss vs. Distance (LOS)	6

3.1 Implementation and Results	6

3.2 Discussion	6

3.3 Conclusion	7

4. Deliverable 2: Channel Selection and Interference Analysis	7

4.1 Method and Implementation	7

4.2 Results and Discussion	7

4.3 Conclusion	8

5. Deliverable 3: Recommended nRF24 Configuration	8

5.1 Discussion on Retry Settings (Latency vs. Reliability)	8

5.2 Conclusion: Final Configuration Table	8

6. Design Communication Protocol for Future Usage	9

List of Figure

Figure 1: Physical hardware setup showing the NodeMCU-32S, ESP32-C3, and the 10 µF/100 nF capacitor modifications on the nRF24L01+ module headers	6

Figure 2: Packet-loss versus distance curves (LOS), plotting 250 kbps, 1 Mbps, and 2 Mbps configurations	7

Figure 3: Channel survey spectrum analysis and Wi-Fi AP interference overlay	9

List of Tables

Table 1: Recommended nRF24 Configuration	11

# 1. Problem Statement

The remote-control dashboard project requires a proven, highly reliable RF communication link. Before finalizing the custom ground-station dongle and the underlying communication protocol, the nRF24L01+ radio modules must be characterized. The core deliverables required to validate this link are packet-loss versus distance curves at multiple data rates, a channel selection strategy based on real-world interference analysis, and a final recommended nRF24 configuration defining the channel, data rate, power, and retry settings.

# 2. System Setup: Hardware Bring-Up and Software Methods

To execute the required characterisation, a standardized test bench was established incorporating both microcontroller hardware and PC-based automation software.

## 2.1 Method (Software Setup)

Instead of relying on manual packet counting, the test methodology utilized a suite of Python automation scripts (run_test.py and channel_analysis.py) executed from a host PC. The PC communicates over a serial port to the receiver (RX) node, which acts as a master controller. The RX node utilizes a fixed, high-power control link (Channel 80) to command the mobile transmitter (TX) node to initiate test bursts using specific configurations. The Python scripts parse the serial telemetry, automatically log the packet error rates (PER) to CSV files, and generate the required performance graphs.

## 2.2 Implementation (Hardware Bring-Up)

The physical architecture deployed an ESP32-C3 development board for the mobile TX node (powered by a USB battery bank) and a NodeMCU-32S for the ground-station RX node. During initial testing, the system suffered consistent brownouts because the PA+LNA modules draw current bursts exceeding 115 mA when operating at maximum transmit power (PA_MAX). This was resolved by implementing a mandatory hardware modification: a 10 µF electrolytic capacitor and a 100 nF ceramic capacitor were added directly across the VCC and GND pins of both nRF24 modules.

Figure 1: Physical hardware setup showing the NodeMCU-32S, ESP32-C3, and the 10 µF/100 nF capacitor modifications on the nRF24L01+ module headers

Figure 1 provides visual proof of the final hardware bring-up, specifically highlighting the decoupling capacitors required to stabilize the 3.3V rail. Additionally, bench testing demonstrated that placing the antennas closer than 1 meter at PA_MAX saturates the receiving Low Noise Amplifier (LNA), requiring the use of PA_LOW for short-range verification.

# 3. Deliverable 1: Packet-Loss vs. Distance (LOS)

To determine the effective range and optimum transmission speed, the link was systematically tested over varying distances with unobstructed line-of-sight.

## 3.1 Implementation and Results

A distance sweep was conducted from 1 meter up to 190 meters LOS. At each distance marker, the Python script commanded the TX node to transmit 1,000 packets at PA_MAX across three data rates: 250 kbps, 1000 kbps, and 2000 kbps. The test measured both raw physical-layer packet loss (NOACK mode) and effective delivery after hardware retries (ACK mode).

Figure 2: Packet-loss versus distance curves (LOS), plotting 250 kbps, 1 Mbps, and 2 Mbps configurations

Figure 2 illustrates the raw packet error rate (PER) over distance for the three tested data rates, demonstrating the inverse relationship between data rate and range sensitivity.

## 3.2 Discussion

## The empirical data clearly maps the performance trade-offs of the three data rates. While the 250 kbps configuration exhibited superior sensitivity at extreme ranges (maintaining a connection up to 190m), it suffered slightly higher raw packet loss at short ranges (5.60% PER at 5m). Conversely, the 2000 kbps rate degraded past 100 meters, but delivered absolutely flawless physical-layer performance at short range, logging a 0.00% raw PER at both 5m and 10m.

## 3.3 Conclusion

The final application requires the mobile node to control a machine on campus from a typical standoff distance of 5 to 10 meters. Therefore, 2000 kbps is the definitive choice for the data rate. At this speed, the packet spends significantly less time on the air, reducing the probability of Wi-Fi collisions. The 190-meter stress tests successfully proved the hardware decoupling is stable under maximum load, but the configuration must be optimized for the 10-meter operational envelope.

# 4. Deliverable 2: Channel Selection and Interference Analysis

Coexistence within the 2.4 GHz ISM band requires strategic channel selection to avoid interference from standard 802.11b/g/n Wi-Fi networks and other project groups.

## 4.1 Method and Implementation

Channel selection was automated via the channel_analysis.py script. The RX node sequentially sweeps all 126 nRF channels to measure ambient RF energy (carrier-detect) while simultaneously performing a standard Wi-Fi access point scan. The script calculates a penalty score for each channel based on a weighted overlap with detected Wi-Fi channels (±11 MHz bandwidth) and ambient RF hits, prioritizing quiet gaps.

## 4.2 Results and Discussion

During testing, the primary campus operating environment yielded 6 overlapping Wi-Fi networks, primarily clustered around Wi-Fi channels 2, 6, and 11. The Python analysis identified a quiet spectral gap at nRF Channel 77 (2477 MHz), which is completely free from high-RSSI Wi-Fi overlap as it sits above the primary Wi-Fi bands.

Figure 3: Channel survey spectrum analysis and Wi-Fi AP interference overlay

Figure 3 graphs the ambient RF energy alongside the detected Wi-Fi networks, visually confirming the algorithm's recommendation of Channel 77. To ensure multi-group coexistence, a minimum guard band of 4 nRF channels must be maintained between active REII links to prevent adjacent-channel interference. The algorithm generated a safe group plan of channels 4, 50, 73, 77, and 81.

To quantify the impact of poor channel selection, a deliberate interference benchmark was run at 5 meters. Testing on nRF Channel 11—which sits directly inside the active Wi-Fi Channel 2 envelope—yielded a raw physical-layer packet error rate of 17.20% at 250 kbps and 20.10% at 1 Mbps. When tested on a cleaner frequency (nRF Channel 4), the raw packet loss dropped to 7.50% and 12.70% respectively. This proves that operating within a Wi-Fi band severely degrades the physical RF link, necessitating the hardware Auto-ACK to work harder to maintain delivery.

Table 1: 5-Meter Interference Benchmark (PA_MAX, NOACK Mode)

| nRF Channel | Environment | 250 kbps PER | 1000 kbps PER | 2000 kbps PER |
| --- | --- | --- | --- | --- |
| Channel 11 | Congested (Inside Wi-Fi Ch 2) | 17.20% | 20.10% | 0.00% |
| Channel 4 | Quiet Spectral Gap | 7.50% | 12.70% | 0.50% |

## 4.3 Conclusion

Based on the environmental sweep and interference benchmarking, nRF Channel 77 (2477 MHz) is selected as the primary operating frequency. It successfully evades campus Wi-Fi interference while remaining within the legal ISM band.

# 5. Deliverable 3: Recommended nRF24 Configuration

The final configuration consolidates the empirical findings into a definitive register map for the upcoming STM32F411 integration.

## 5.1 Discussion on Retry Settings (Latency vs. Reliability)

While configuring the Auto-ACK retries, an engineering trade-off exists between reliability and latency. To maintain real-time dashboard responsiveness, excessive retries must be avoided. A single transmission attempt at 250 kbps (32-byte payload, 1.5 ms retry delay) takes approximately 4.1 ms. Setting a maximum limit of 5 retries (ARC = 5) caps the worst-case radio delay at 20.5 ms. This securely covers the maximum 3 retries observed during the 190-meter field tests while ensuring data latency remains negligible for the operator.

## 5.2 Conclusion: Final Configuration Table

Table 1 outlines the mandated hardware parameters derived from the characterisation phase.

Table 2: Recommended nRF24 Configuration

| Parameter | Selected Value | Engineering Justification |
| --- | --- | --- |
| Channel (RF_CH) | 77 (2477 MHz) | Located in a quiet spectral gap (74–81) strictly above the congested campus Wi-Fi infrastructure (Wi-Fi channels 1, 6, 11) |
| Data Rate (RF_DR) | 250 kbps | Maximized receiver sensitivity; achieved 0.00% effective PER at 190m LOS. |
| PA Level (RF_PWR) | PA_MAX | Necessary for long-range operational telemetry (must be reduced to PA_LOW for < 2m bench tests). |
| Retry Count (ARC) | 5 | Absorbs the worst-case packet loss (3 retries measured) while strictly capping latency at ~20.5 ms. |
| Retry Delay (ARD) | 1500 µs | Hardware constraint; minimum required time to process 32-byte ACK payloads at 250 kbps. |
| Address/CRC | 5 bytes / 16-bit | Standard robustness configuration. |

# 6. Design Communication Protocol for Future Usage

The final objective of this phase is to dictate how the STM32F411 and application protocol will interact with the RF link.

Based on the highly successful ACK mode performance documented in Deliverable 1, implementing a complex Automatic Repeat Request (ARQ) scheme at the software application level is deemed unnecessary and constitutes over-engineering. The nRF24L01+'s intrinsic Enhanced ShockBurst hardware Auto-ACK feature is perfectly capable of maintaining link integrity, successfully masking physical-layer drops up to 190 meters.

The future STM32 firmware should rely strictly on the hardware configuration defined in Table 1. The communication protocol will utilize standard data packets for downstream commands from the dashboard, and leverage ACK-payloads (piggybacking data onto the automatic hardware acknowledgments) for upstream telemetry from the mobile node. This approach minimizes overhead, prevents unnecessary application-layer timeouts, and officially validates the RF hardware architecture for the next phase of development.